WANPIPETM

Multi-protocol WANPIPE Driver for Linux®

DEBUGGING M A N U A L

Author: Nenad Corbic

Copyright (c) 1995-2002 Sangoma Technologies Inc.

# Introduction

At the heart of WANPIPE debugging is the 'wanpipemon' debugging program.  It is used to display line traces, protocol statistics, hardware statistics and configuration parameters.

/usr/sbin/wanpipemon -g

# Automated Debugging

The wanrouter operation script can retrieve the most common line error statistics that can give the user a good idea of what is going on with the line.

/usr/sbin/wanrouter debug if_name

or

/usr/sbin/wanrouter debug if_name > debug_file.txt

(where if_name is the name of wanpipe interface: eg: wp1fr16)

# 'wanpipemon' usage

/usr/sbin/wanpipemon

Wanpipemon Verison 1.0

Copyright (C) 2002 - Sangoma Technologies

www.sangoma.com

This may be freely redistributed under the terms of the GNU GPL

Usage:  (Options in {} brackets are optional)

#Command prompt Local debugging

wanpipemon -i <interface> -c <command> { -p <protocol> }

#Command prompt Remote debugging

wanpipemon -i <remote IP> -u <port> -c <command> { -p <protocol> }

#GUI interface

wanpipemon -g { -i <interface|IP> -u <port> -p <protocol> }

#Display usage for each protocol

wanpipemon -p [fr|ppp|chdlc|x25|adsl]

#Detailed usage for each option

wanpipemon -h

## Local debugging:

The 'wanpipemon' utility is located on the same

machine as the WANPIPE card.

Thus, the wanpipemon utility needs only the

interface name (eg: wp1_fr16) to reach the driver.

/usr/sbin/wanpipemon -i wp1_fr16 -c xm

## Remote debugging:

The 'wanpipemon' utility is located on a network

(LAN or WAN) and must use the UDP protocol to

reach the WANPIPE card.

In this case an IP and UDP addresses must be

specified, such that the packet will pass through

the WANPIPE driver!

Meaning a POINTOPOINT address must be used instead

of the LOCAL IP address.

The UDP port was specified in WANPIPE configuration

file. (Default=9000)

/usr/sbin/wanpipemon -i 201.1.1.2 -u 9000 -c xm

## GUI Mode Operation

The GUI mode is based on ncurses libraries, thus it

will work both in Terminal or Xwindows mode.

/usr/sbin/wanpipemon -g

Without any other arguments the GUI will offer a list

of network interfaces, for local debugging, or will

ask for an IP/UDP address, for remote debugging.

Use menu's to navigate through available commands.

# Common Wanpipemon Statistics

## Modem Status (-c xm):

wanpipemon -i wp1fr16 -c xm

Must be DCD=HIGH and CTS=HIGH if not there is

either a cable problem or the sync line is

not turned ON.

## Communication Error Stats (-c sc):

wanpipemon -i wp1fr16 -c sc

Check for CRC and Abort errors. If errors

are incrementing, there is a line issue.

Possible reasons: CSU/DSU misconfiguration

on local or remote side.

Noisy/Bad line.

## Line trace (-c tr):

wanpipemon -i wp1fr16 -c tr

Make sure thare are incoming and outgoing frames.

If frames are outgoing and none incoming:

Frame relay: Check signalling (ANSI, LMI)

Make sure ISP uses IETF or CISCO

data encapsulation.

Other prot:  Possible CRC errors in TX

direction. Contact your ISP.

If frames are both incoming and outgoing but

protocol is still down:

Run trace in Interpreted mode (-c ti) and

make sure that the incoming protocol is

indeed the one you are configured for.

# Wanpipe Debugging

## Wanrouter Log File

wanrouer conflogs  /   cat /var/log/wanrouter

All wanpipe configuration file syntax errors will be displayed in wanrouter log file.  Thus, if wanrouter commad returns a syntax error, the description of the error will be generated in /var/log/wanrouter log file.

## Kernel Log File

wanrouter messages			#Display all wanpipe messages

or

tail -f  /var/log/messages    		#Display last ten lines

All wanpipe, kernel module events and errors will be displayed in syslog.  The most common syslog file is  /var/log/messags.

## Sangoma Tech Support

When reporting problems to Sangoma Tech Support please dump the current system debug information into  a temporary file and send it via email.

- wanrouter debug wp1fr16 > sangoma_debug_file.txt

- Wait for 2-5 minutes

- Stop the debug by pressing ENTER

- Email sangoma_debug_file.txt to Sangoma Tech Support

The above diagram displays a Wanpipe line debugging flow chart.  Each flow chart command (eg: Tv) is a /usr/sbin/wanpipemon command.  

Thus, if the chart indicates that statistics for command xm are required, a full command would look like the following:

/usr/sbin/wanpipemon -i if_name -c xm