WANPIPETM

Multi-protocol WANPIPE Driver for Linux®

INSTALLATION   M A N U A L

Author: Nenad Corbic

Copyright (c) 1995-2002 Sangoma Technologies Inc.

# Introduction

The WANPIPE Routing/API solution consists of Sangoma "S" series intelligent adapters/cards, Linux kernel device drivers and shell/GUI configuration and startup scripts.  The Sangoma "S" adapters along with the WANPIPE device drivers provide a physical and WAN data link layer (OSI model) to the Linux IP Routing Stack.   The Linux IP stack, using its internal routing table, routes packets to and from the WANPIPE drivers.

# WANPIPE Distribution

Wanpipe packages are distributed via FTP ftp.sangoma.com/linux and CDROMs that are shipped with Sangoma Wan cards.  Due to dynamic nature of Linux kernels, the CD-ROM Wanpipe distributions usually became outdated in a short period of time.  It is recommended to always upgrade to the latest STABLE release when installing Wanpipe.

# WANPIPE Packages

Wanpipe drivers are packaged in Source (.tgz) and Binary. Binary packages are in RPM (.rpm) and LRP (.lrp) formats.

## Wanpipe RPMS

Wanpipe RPM packages are located in: ftp.sangoma.com/linux/RPMS or distributed CD's.

Please refer to RPM package installation below.

## Wanpipe Source

Wanpipe SOURCE packages are located in: ftp.sangoma.com/linux/current_wanpipe or distributed CD's.

Download the latest SOURCE wanpipe release 
in ANY directory:

ex: cd /tmp
	tar xvfz wanpipe-<version>.tgz 

	The tar utility will create a temporary wanpipe/ directory.

## Wanpipe LRP

Wanpipe LRP packages are located in: ftp.sangoma.com/linux/LRP or distributed CD's

IMPORTANT:  Current wanpipe packages only support cards S508/S514 and greater.

PLEASE CONTACT SANGOMA TECHNOLOGIES INC. IF YOU WANT TO UPGRADE YOUR OLD S502E or S503 CARD.

# Choosing the Kernel Version

Wanpipe drivers are being kept in sync with the latest kernels.
Although, Wanpipe will run with all kernel versions, we recommend using the stable kernels. 

i.e. Stable kernel versions that contain only even numbers.

Eg: 2.4.18 = stable      2.4.19 = development

# Wanpipe RPM Installation

## Automated RPM Setup

Download RPM Setup installation script from: ftp.sangoma.com/linux/RPMS 

	./Setup install <abs path>

Setup installation script will scan you system and suggest an RPM.
If the <abs path> is not supplied:

Setup will assume that it's part of a CD distribution.  It will expect the RPM directory structure as on the CD or FTP site.

	e.g.: RPMS/<distr>/<version>/

If the RPMS are not found locally, Setup can 
download the latest releases from the Sangoma FTP site.

Once the RPM is downloaded, Setup can proceed 
to install the RPM for you.

## Manual RPM Setup

- Determine the Linux Distribution:
 	
	cat /etc/issue
 	(e.g.: RedHat 7.1)

- Determine the Kernel image you are running:
 
 	uname -r

 	(e.g.: 2.4.2-2: 	it is not an smp kernel)
 	(e.g.: 2.4.2-2smp: 	smp kernel)

- Determine the processor type
 
	uname -m
 	(e.g.: i686)

- Based on the above information choose an RPM package.

	wanpipe-<version>.<linux distrib>.<kernel ver>.<processor>.rpm

	e.g.: ftp.sangoma.com/linux/RPM/redhat/7.1/i686/wanpipe-2.2.4-3.rh71up.k242-2.i686.rpm

- Download the RPM package.

- List the contents of the rpm

rpm -qipl <wanpipe rpm name>

Make sure that the /lib/modules/<kernel name> directory
matches your current kernel modules directory.

i.e. the output of command 'uname -r' must match the above 	<kernel name> directory.

- The rpm must be installed using the --force option, to override the old modules.

 rpm -i --force <wanpipe rpm name>

- Read 'README-2.config' on how to configure wanpipe device

# WANPIPE Source Package Setup

The Setup installation script, located in wanpipe temporary directory, will guide you through either installation procedure, whether it's a fresh install or an upgrade.  The installation utility will probe the kernel for previous WANPIPE installations and will offer appropriate installation options.

Furthermore, the Setup installation utility can also re-compile and install new WANPIPE device driver modules based on the currently running Linux kernel, thus eliminating manual kernel/module recompilation.

To start WANPIPE installation, run the Setup installation program.

cd wanpipe/

./Setup install

This script will perform the following functions:

- WANPIPE INSTALLATION

Verify distribution integrity and fix file permissions
Check for required packages, bash2, libncurses and GCC.

Note: Wanpipe GUI utilities require BASH version 2 or greater.

If the Setup fails the bash check, make sure bash in installed before attempting to configure WANPIPE.

- UPDATING WANPIPE DEVICE DRIVERS: LINUX KERNEL

Probe the Linux kernel for previous WANPIPE installation
If the kernel is clean, appropriate patches will be used 
updated WANPIPE drivers.

If the kernel already has WANPIPE installed, drivers will be upgraded to the latest version.

- WANPIPE KERNEL DRIVER SETUP

This option will attempt to compile currently installed WANPIPE device
drivers located in the Linux source tree and update/install WANPIPE
modules.
Note: kernel source in /usr/src/linux must have the same configuration and version number of the currently running image.  Otherwise, module dependencies will fail.

- WANPIPE BOOTSTRAP CONFIGURATION

Option to install bootstrap startup and shutdown scripts that will start and stop WANPIPE drivers on system startup and shutdown.

- WANPIPE META CONFIGURATION 

Wanpipe utilities depend on a wanrouter.rc META file that defines locations of WANPIPE lock, configuration and interface files. 

You will be prompted to define these paths.  If unsure leave everything as default.

- WANPIPE UTILITES SETUP

This option will compile all WANPIPE utilities necessary to run WANPIPE devices.

- WANPIPE INSTALATION : COMPLETE

This is the last message in the installation process.  This message will ONLY be displayed if the WANPIPE device driver modules have been 	compiled.   In this case, the next step is to configure a WANPIPE driver and start the router.

PLEASE SKIP THE NEXT SECTION AND PROCEED TO WANPIPE CONFIGURATION.

- WANPIPE INSTALLATION: INCOMPLETE

This is also the last message in the installation process.  This message will ONLY be displayed if the WANPIPE device drivers HAVE NOT been compiled, thus you are required to proceed with the kernel/module re-compilation in order to finish up the WANPIPE installation.

PLEASE REFER TO THE LINUX COMPILATON INSTRUCTIONS BELOW

Please run: ./Setup -help for further help.

## Customizing Wanpipe Source Package

##### Installation using the '--silent' option

The '--silent' option is used to install wanpipe without any user interaction:

	./Setup install --silent

All defaults will be selected:

- Configuration directory /etc/wanpipe

- Not Enable IP Forwarding on startup

- Start wanpipe on boot up

- Recompile modules for the kernel source in /usr/src/linux

##### Installation using the --builddir option

When customizing the wanpipe package, it is useful to install the package into a virtual directory so not to pollute the current environment.

	e.g. ./Setup install --builddir=/tmp/wanpipe_build

Note: the /tmp/wanpipe_build directory must exist.

##### Installation using the --protocol option

Standard Wanpipe installation will only include MAINSTREAM WAN protocols when building the kernel modules to cut down on kernel module size.

Mainstream Protocols: 
	Frame Relay, PPP, CHDLC, X25, API stack

To enable NON Mainstream protocols specify extra protocols with the --protocol option.

Non-Mainstream Protocols:

BSC		: API for Multipoint Bisync

BSCSTRM 	: API for Bisync Streaming

SS7		: API for Level2 SS7 protocol

EDU 		: API for Educational software

SDLC		: API for SDLC protocol

e.g: ./Setup install --protocol=BSCSTRM

##### Building a custom RPM

./Setup buildrpm

All defaults will be used:

- Configuration directory /etc/wanpipe

- NOT Enable IP Forwarding on startup

- Start wanpipe on boot up

- Recompile modules for the kernel source located in /usr/src/linux directory,
or directory supplied with --with-linux=<abs path>	 argument.

Refer to README.rpmbuild

##### Installation using the  -drivers option

This option should be used to patch a new kernel source, or to upgrade a kernel source with the current version of the WANPIPE drivers.

NOTE: If this is a first time install you MUST run the install option: Setup install.

Run the Setup script in temporary wanpipe/ directory:

e.g.: cd wanpipe/
./Setup drivers

The drivers option will do the following:

- Prompt the user to Install/Upgrade Wanpipe drivers into the Linux kernel source.

NOTE: The kernel source must be located in /usr/src/linux

- Prompt the user to re-compile all WANPIPE kernel drivers and install the as modules in the current /lib/modules/<kernel-version>  directory.

##### Installation using the  -utility option

This option will re-compile all WANPIPE utilities in temporary wanpipe/ directory.

NOTE: If this is a first time install you MUST run the install option: Setup install.

Run the Setup script in temporary wanpipe/ directory:

ex: cd wanpipe/
./Setup utility

The utility option will do the following:

Compile all WANPIPE utilities and install them
in /usr/sbin directory.

For more info about Setup command run ./Setup without any arguments for full usage info.

## WANPIPE Source Package Un-Install

Run the Setup script in temporary wanpipe/ directory:

ex: 
cd wanpipe/
./Setup remove

The remove option will do the following:

- Remove all bootstrap WANPIPE scripts.

- Remove all utilities from /usr/sbin, /etc/wanpipe, /usr/lib/wanpipe/, and   /usr/doc/wanpipe directories.

After the 'Setup remove' command, it is safe to remove the whole temporary wanpipe/ directory.

e.g:   rm -rf wanpipe/

# LRP Binary Installation

###### Package Contents:

###### wanpipe.lrp:   Wanpipe utilities and startup scripts 
modules.lrp:   Wanpipe kernel modules

- Copy the lrp packages onto a DOS formatted disk.

- Boot the system with the LRP boot up disk

Insert the disk containing Wanpipe lrp packages
Mount the disk:  e.g.: mount.boot /mnt
Change directory to /mnt and install each package

E.g.: 	lrpkg -i modules
 		lrpkg -i wanpipe

- Create the Wanpipe configuration fine in /etc/wanpipe directory.

# Linux Kernel Configuration/Compilation

If Wanpipe kernel modules were recompiled by the Setup installation script, this step is not necessary!  

For further, kernel installation, information refer to /usr/src/linux/README.

## Configure Linux Kernel

Change your current directory to /usr/src/linux: 
	make menuconfig 		(menu- driven configuration) 
or 
	make xconfig		(if you are running X-Windows.)

Note:

If you have never configured Linux kernel before, please read /usr/src/linux/README or ask someone who did for assistance.  In short, the Linux configuration script will ask you a series of questions regarding your system configuration.  Most questions require single-letter answer: 'y' to enable feature, 'n' to disable it or 'm' to implement feature as a module.  Entering 'h' brings up a help text (if available).

IMPORTANT:

When configuring the kernel, MAKE SURE YOU ANSWER NO to the following questions when they come up:

Wan Interfaces

Frame relay DLCI support (CONFIG_DLCI)

When configuring kernel, MAKE SURE YOU ANSWER YES or M to the following questions when they come up:

Code maturity level options

Prompt for development drivers	(CONFIG_EXPERIMENTAL)

Loadable Module Support

Enable loadable module support	(CONFIG_MODULES)

Kernel daemon support		(CONFIG_KMOD)

General Setup

Networking support			(CONFIG_NET)

PCI support				(CONFIG_PCI)

Networking Options

Packet Socket			(CONFIG_PACKET)

Unix domain sockets		(CONFIG_UNIX)

TCP/IP networking			(CONFIG_INET)

IP: optimize as router not host	(CONFIG_IP_ROUTER)

802.1d Ethernet Bridging		(CONFIG_BRIDGE)

WAN Router				 (CONFIG_WAN_ROUTER) set to "M"

Network Device Support

Network device support		(CONFIG_NETDEVICES)

Wan Interfaces

WAN Drivers				(CONFIG_WAN_DRIVERS)

Vendor Sangoma			(CONFIG_VENDOR_SANGOMA)

In previous version of WANPIPE, a maximum number of WANPIPE devices needed to be defined (CONFIG_WANPIPE_CARDS).  In this release, all Sangoma devices are probed during module startup.  Devices found are dynamically setup.

WANPIPE Frame Relay support (CONFIG_WANPIPE_FR)

Say 'Y' to this option, if you are planning to connect WANPIPE card to a Frame

Relay network.  If you say 'N', the frame relay support will not be included in the driver (saves about 16K of kernel memory).

WANPIPE Cisco HDLC support	(CONFIG_WANPIPE_CHDLC)

Say 'Y' to this option, if you are planning to connect WANPIPE card to a leased line using Cisco HDLC protocol (WANPIPE or API).  If you say 'N', the Cisco HDLC support will not be included in the driver.

WANPIPE PPP support		(CONFIG_WANPIPE_PPP)

Say 'Y' to this option, if you are planning to connect WANPIPE card to a leased line using PPP protocol (WANPIPE).  If you say 'N', the PPP support will not be included in the driver.

WANPIPE X25 support		(CONFIG_WANPIPE_X25)

Say 'Y' to this option, if you are planning to connect WANPIPE card to a leased line using X25 protocol (WANPIPE or API).  If you say 'N', the X25 support will not be included in the driver.

WANPIPE Multi-Port PPP support	(CONFIG_WANPIPE_MULTPPP)

Say 'Y' to this option, if you are planning to connect WANPIPE card to multiple leased line, using the kernel PPP protocol (WANPIPE).  Since the protocol is implemented in the kernel PPP protocol can run on both adapter ports.  If you say 'N', the Multi-Port PPP support will not be included in the driver.

Filesystems

/proc filesystem support		(CONFIG_PROC_FS)

## Re-build Linux kernel

If you never compiled Linux kernel before, please read   /usr/src/linux/README or ask someone who did for assistance.  Note that if your machine is slow and/or has little memory, re-building Linux kernel may take several hours.

After successfully configuring the kernel using 'make menuconfig' perform the following (from /usr/src/linux directory):

make dep	:to rebuild source code dependencies,

make clean	:to delete all stale object files,

Note:

Save your '/usr/src/linux/.conf' file to some other location and rename it to (ex: conf_old).  This file contains your linux configuration; thus, if you ever have to reconfigure a fresh kernel, copy this file back into the linux directory, rename it to '.config' and run 'make menuconfig'.

make bzImage 	(2.2.X / 2.4.X kernels) to build new kernel.

IMPORTANT:

Before we proceed to build new modules, it's advisable to remove the old, already existing modules.

Change directory to /lib/modules

Rename the "kernel_name" directory to "kernel_name.old".

(Ex. If we are working with kernel 2.2.14, then rename the 2.2.14 directory to 2.2.14.old)

Change directory back to /usr/src/linux.

make modules		to build kernel modules,

make modules_install	to install modules.

## Updating LILO

This section only applies if the kernel was compiled using bzImage or zImage.

After successful compilation, copy the new image usr/src/linux/arch/i386/boot/zImage (or bzImage which ever was compiled)  into the /boot directory and rename it to vmlinuz_2.2.X (where X is the kernel number.)

(ex. cp /usr/src/linux/arch/i386/boot/bzImage /boot/vmlinuz-2.2.14)

Note:

Before you copy the new kernel into the /boot directory rename the old kernel image in /boot.  (ex. mv vmlinuz-2.2.10 vmlinuz-2.2.10.old)

In /etc directory, edit the lilo.conf file and update the changes. For more info run 'man lilo'.   

	lilo	:execute lilo to update the changes to /etc/lilo.conf

When all this is done, reboot your machine to load the new image !!!