# X25API Programming Manual

Date: July 31, 2026

Author: Nenad Corbic

## Introduction

The new X25API driver has been completely redesigned to take advantage of Linux socket architecture. Due to unsecured nature of standard Linux sockets, Sangoma has developed a secure streaming socket which guarantees packet delivery during high traffic.

Application Programming Interface (API), is used to send and receive custom raw, non-IP, packets to and from the card.   Since data communicated is not in IP format, the network interface is configured without the IP address.  This effectively removes the kernel routing table entry and unhooks the IP routing stack from the Wanpipe driver.  Non-IP communication is achieved using the RAW socket calls to the driver.  As the name implies, packets are transferred without any modification.

Most API applications are very sensitive to packet loss, which presented a problem in using Linux Raw sockets, which silently dropped packets during high traffic.  Dropping packets is ok for TCP/IP but not for X25:  protocol based on zero drop policy.

Consequently, the secure socket solution was created; custom socket that guaranteed delivery in both upstream and downstream. The Wanpipe socket is based on the Linux raw socket, developed by Alan Cox and others.  Following rules were needed to guarantee delivery:

- Check for adequate buffer space before sending data.

- If send buffers are full the user is blocked from sending until buffers are freed up.

- If the receive sock buffers are full, driver will enable protocol flow control to slow down traffic until receive buffers are freed up.

## Installation

X25API and Wanpipe Socket drivers, are included in the latest WANPIPE package, (refer to ftp site below).

Please refer to WANPIPE user manual:  WanpipeInstallation.pdf,

It contains Wanpipe package installation, compilation and startup.

Location:  Web:	ftp.sangoma.com /linux/current_wanpipe/doc.

Local: 	/usr/share/doc/wanpipe/

Details about structures, commands and return codes can be found in the manual entitled “X.25 SUPPORT SANGOMA CARDS”; February 2000. (/usr/lib/wanpipe/doc/x25.pdf)

## Configuring X25API

Once the wanpipe package is installed, proceed to create a /etc/wanpipe#.conf configuration file, which will be used to startup and configure wanpipe drivers/protocols.

Configuration utility:  /usr/sbin/wancfg

Instead of manually configuring the wanpipe configuration file, use a GUI

configuration utility, ‘/usr/sbin/wancfg’.  The wancfg application contains all the help files needed to successfully create a wanpipe configuration file.

In case you need more information refer to WANPIPE configuration manual: WanpipeConfiguration.pdf

Located:  Web:	ftp.sangoma.com /linux/current_wanpipe/doc

Local:  	/usr/share/doc/Wanpipe/

## Starting up X25API

Once the wanpipe configuration file is created, start up the x25api driver using “wanrouter start” command.

- /usr/sbin/wanrouter start wanpipe1

- Check /var/log/messages file for configuration errors and link status.

Before attempting to send data, make sure the “link connected” message is present in the message file. If the X25/HDLC protocol layer is not connected, no communication is possible.

For further debugging and line testing, use the xpipemon debugged and refer to the WANPIPE operational manual: WanpipeOperation.pdf

Located:  Web:	ftp.sangoma.com /linux/current_wanpipe/doc

Local:  	/usr/share/doc/Wanpipe/

## Wanpipe API Architecture

The X25API interface uses sockets to communicate to each x25 logical channel (svc or pvc).  Standard TCP/IP calls must be used to setup and establish connection to the remote server/client.

### Socket Handling

- Once the connection is established, each socket acts as a data pump transmitting data to and from the driver.  Since the socket buffering is limited (64KB), the user application is responsible to pull data out of the sockets as fast as possible.

- If the single channel, on the x25 link, fails to send data up to the socket due to socket being full, the driver will implement x25 flow control.  Which will block the remote side from transmitting until the user application has pulled packets out of the full socket.

- Thus, on a multi channel x25 link, if a single channel is ignored, and the socket for that channel becomes full, the x25 flow control will block all incoming that for that particular link.   In other words, a single channel can block ALL active channels on an x25 link.

  - To avoid this from happening, a select() system call must be setup to wait on all active sockets.   So all events from all active channels, regardless of the user application states, are handled.

### Event handling

- Once the communication is established, the application uses the socket to transmit and receive data to and from the remote connection.

- The channel will stay connected until the out of band (oob) event is received, or the user initiates disconnect (clear call).

- The OOB event must be decoded, and handled by the application (i,e,  application should handle its states accordingly).  Not all OOB events will bring the connection down (Reset and Interrupt packets). The user should check the state of the socket before closing it.

- It is very important to pull off any remaining RX data out of the socket, after the OOB event has occurred, before closing the socket.  Otherwise, the close() system call will delete all socket queues as well as the socket itself.

User Space

Kernel Space

Place Call

Send a place call command down to the driver and wit for accept.

(if non-blocking is set, return)

Set sock state to listen

Make sure that sock has been bounded to device listening port.

Create Socket

Allocate and init sock structure.

Bind sock to net interface

API Command Execute

Execute cmd and wait for response in case of blocking. Otherwise return result.

Wanpipe Secure Sock

Wanpipe Device Driver

Bind sock to driver

Find a free channel/interface and bind a sock to it.

Rx  listen queue.

Create new sock

Bind to incoming chan

Rx Call Pkt

Put call in listen queue.

Incoming Call

Create a event pkt and send up the listen sock.

X25 API Application

socket() bind() connect() listen(), accept()

User Space

Kernel Space

X25 API Application

rcv(), send(), select()

Send

Check if driver is busy

Pass a packet down to the driver

Select function, Poll

Check tx queue, if full block send()

Check rx queue, if data trigger rcv()

Check error queue, trigger rcv_oob()

Receive Data

Put the packet into a rx queue

Receive OOB

Put error into a error queue

Rcv or OOB

De-queue rx  or oob, and send to user.

Send

Write to adapter

Rx_intr

Put the packet on  api queue.

Event_intr

Pass OOB packet up the stack

Rx BH

Pass data up the stack. If full, wait and try again. Enable flow control.

Wanpipe Secure Sock

Wanpipe Device Driver

## X25API Client Programming Model

The x25api client is responsible for initiating outgoing calls to the server.

Once the call is accepted, the client proceeds to send and receive data packets over the x25 network.   The communication will continue until the client initiates a disconnect or an out-of-band (oob) event occurs (eg. Clear call).   The example code /usr/lib/wanpie/api/x25/svc_client.c should be used to kick start the x25api client development.

  - Create a socket

    - select() system call

  - Bind socket to a free svc network interface

    - bind () system call

    - device name “svc_connect”

  - Write call data information into the socket.   					(-d<called address> -s<calling addr> -f<facils> -u<user data>)

    - ioctl(SIOC_WANPIPE_SET_CALL_DATA);

- Initiate outgoing call to the server

  - connect() system call

    - Block method:  		waits for the remote response

    - Non-Block method: 	returns as soon as call is placed.

- Initialize the select() statement and block for receive, transmit and oob.

  - select() system call

- Proceed to send and receive data

- Once an oob connection is received, or the connection is terminated, the socket  MUST be closed.

## X25API Server Programming Model

The x25api server is responsible for accepting and handling incoming connections.  Once the server accepts a call, a child process should be spawned to handle the data tx/rx and oob events for that particular channel.   The server process must remain in listening mode waiting for incoming call requests.   The child will continue to exist until the connection is up.  Once the OOB message is received, the child must handle the OOB message, close the socket and die.

The child can be created using a fork(), clone() or pthread.  The fork() system call is adequate for small number of channels (1-50), however its very expensive and is not very scalable for application running 100+ simultaneous connections.   The pthread architecture is the fastest and most scalable method of implementing a server.  The x25api sample codes contain three implementations of the x25api server, located in /usr/lib/wanpipe/api/x25 directory.

server_v1.c   : fork() based server

servier_v2.c  : single process server, that doesn’t spawn children

pthread/server.c : pthead based server that uses threads to spawn children.

  - Create a socket

    - select() system call

  - Bind a socket to a x25 link listening port

    - bind () system call

    - device name “svc_listen”

  - Set the socket in listening mode

    - listen() system call

  - Setup select system call to wait for any incoming call requests on the listen socket.

    - select(listen_sk_id,&readwait,NULL,NULL,NULL)

  - When an incoming call arrives use the accept() system call to create a new socket, which will be used to tx/rx data to and from the remote connection.

    - accept() system call

  - The server must send an accept to the remote clinet in order to establish a connection.  This can be done by the server or by the child which will be handling that connection.  It is suggested that the child sends the accept call, so that the server can go back to listening faster.

  - Once the call is established the server should tx/rx data until an OOB event occurs or disconnect transmitted (clear call).

## X25API System Calls

X25API programming model is closely modeled to TCP/IP socket programming.   Functions such as, sock(), bind(), listen(), accept() and connect() are used to communicate to the X25API driver.

The /usr/lib/wanpipe/api/x25 directory contains the samples code for the server and client implementations. We suggest that you edit these files and use them as a starting point in your x25api application development.

a. socket() function

int socket(int domain, int type, int protocol);

The socket function call creates a new sock and returns

the sock file descriptor to the user. This must be done before any other

BSD IPC system call is executed.

The returned file descriptor is used for the subsequent

system calls used to establish an SVC or PVC connection.

Domain: AF_WANPIPE

Type: SOCK_RAW

Protocol: 0

Ex: int sock = socket(AF_WANPIPE, SOCK_RAW, 0);

Please refer to the sample programs server.c and client.c in

/usr/lib/wanpipe/api/x25 directory.

b. bind() function

int bind ( int sock fh, struct sockaddr *addr, int *addrlen);

After a socket has been created, sock must be bound to a network device.

The bind() system call binds the newly created sock to the appropriate network

device. Only after the sock has been bound, can communication between the sock and the driver take place.

During driver startup, a network device is created for each LCN; defined in

WANPIPE configuration file.  Bind function uses a wan_sockaddr_ll address structure to bind a socket to the appropriate network device.

Thus, the wan_sockaddr_ll structure must be filled in and passed by reference to the bind system call.

struct wan_sockaddr_ll

{

unsigned short sll_family;

unsigned short sll_protocol;

int sll_ifindex;

unsigned short sll_hatype;

unsigned char sll_pkttype;

unsigned char sll_halen;

unsigned char sll_addr[8];

unsigned char sll_device[14];

unsigned char sll_card[14];

};

- Server Process: SVC

struct wan_sockaddr_ll addr;

addr.sll_family = AF_WANPIPE;

addr.sll_protocol = htons(X25_PROT);

strcpy(addr.sll_device, “svc_listen”);

strcpy(addr.sll_card, “wanpipe1");

svc_listen: Represents a virtual network interface name, which

is used to bind a sock to a listening queue for a

particular wanpipe card.

wanpipe#: 	    Wanpipe card name. It is used to bind a sock to a

correct WANPIPE card. This prevents conflicts in

multiple card systems.   # = 1,2,3 ... 16 : Card

number

ii. Client Process: SVC

struct wan_sockaddr_ll addr;

addr.sll_family = AF_WANPIPE;

addr.sll_protocol = htons(X25_PROT);

strcpy(addr.sll_device, “svc_connect”);

strcpy(addr.sll_card, “wanpipe1");

svc_connect: Represents a virtual network interface name,

which is used to bind a sock to a next available

network device on a particular wanpipe card.

wanpipe#:       Wanpipe card name. It is used to bind a sock to a

correct WANPIPE card. This prevents problems in

multiple card systems.  # = 1,2,3 ... 16 : Card

number

Please refer to the sample files, server_v1.c and svc_client.c in

/usr/lib/wanpipe/api/x25 directory.

c. listen() system call

int listen ( int sockfh, int backlog);

The listen() system call prepares a socket to receive CALL INDICATION packets.  All CALL_INDICATION packets are put into this queue for a particular Wanpipe card. The server cannot receive a connection request until it has executed a listen() system call.

After a socket has been set into a listening mode, it cannot be used to transmit or

receive data, it can only be used to accept incoming calls.

Refer to the /usr/lib/wanpipe/api/x25/server.c example code.

d. accept() system call

int accept ( int sock fh, struct sockaddr *addr, int *addr_len);

Before accept() system call can be executed, the socket be in listening mode, otherwise an error will be generated.

The accept() system call returns a socket descriptor for a socket associated

with an SVC connection. The accept() system call doesn’t establish the x25

connection, instead it’s up to the user to analyze call data and respond accordingly:	accept or reject the call.

The accept() call blocks the socket until a CALL REQUEST packet arrives.

The addr and addrlen fields in accept() system call are optional, they can be set to NULL.

Up on a successful CALL INDICATION, the addr structure will contain the

network device the call came in on. This information is not very useful thus, it is

recommended that the accept() call is used with addr and addrlen fileds set to NULL.

Ex: int sock1 = accept(sock, NULL, NULL);

where sock variable is a file descriptor of the listeing socket.

To establish a connection, the user application must execute an ACCEPT command through an ioctl call

Ex: int ioctl(SIOC_WANPIPE_ACCEPT_CALL ,&accept_data);

Please refer to the IOCTL section of the manual.

e. connect() system call

int connect ( int sock fh, struct sockaddr *addr, int *addrlen);

Once a socket was bound to a “svc_connect” virtual network device (refer to

bind() system call), client application may request the X.25 connection using

connect() system call.

Before connect() system call, the user must supply, call data information through an IOCTL call.

int ioctl (sock,SIOC_WANPIPE_SET_CALL_DATA,&data);

The “data” structure is as follows: “x25api_t data”;

typedef struct {

unsigned char qdm PACKED; /* Q/D/M bits */

unsigned char cause PACKED; /* cause field */

unsigned char diagn PACKED; /* diagnostics */

unsigned char pktType PACKED;

unsigned short length PACKED;

unsigned char result PACKED;

unsigned short lcn PACKED;

char reserved[7] PACKED;

}x25api_hdr_t;

typedef struct {

x25api_hdr_t hdr PACKED;

char data[X25_MAX_DATA] PACKED;

}x25api_t;

Therefore, before the ioctl() call is executed, fill in the call data information in the

above structure. Please refer to the sample client application in

/usr/lib/wanpipe/api/x25 directory.

x25api_t api_data;

memset(&api_data,0,sizeof(x25api_t));

sprintf(api_data.data, “-d1234 -s2345 -f3232 -uC21010");

api_data.hdr.length = strlen(api_data.data);

Once the call data has been properly set, we can request x25 link establishment

using the connect() system call.

int connect(sock, NULL, NULL);

On a successful call establishment, the addr structure will contain the network

device on which the connection took place. This information is not very useful;

thus, it is recommended that addr and addrlen variables are set to NULL.

f. Socket ioctl() system calls

int ioctl (int sock_fh, int x25api_cmd, unsigned long data);

x25api ioctl commands are as follows:

SIOC_WANPIPE_SET_CALL_DATA :

Set the call information data into a socket mailbox. This mailbox is used to

send the command down to the driver.

This command must be executed, before a client requests connection

establishment using connect() system call.

SIOC_WANPIPE_GET_CALL_DATA:

Get the incoming call data for the socket mailbox. Once accept() system

call returns a new socket descriptor, the above commands must be executed to retrieve the incoming call information. Then it is up the the user to accept or clear the pending call.

SIOC_WANPIPE_ACCEPT_CALL:

Accept the incoming call. Once the incoming call data has been approved,

the above command will establish the call. Note, that data filed is optional

for this command, thus, data filed can be set to zero.

SIOC_WANPIPE_CLEAR_CALL:

The user can clear the call at any time using the above command. The data filed is optional; Thus, it can be set to zero in most cases.

SIOC_WANPIPE_RESET:

The user can send a reset any time once the connection is established. The data filed is optional; It can be set to zero in most cases.

SIOC_WANPIPE_INTERRUPT:

The user can send an interrupt packet any time once the connection is established.

SIOC_WANPIPE_SET_NONBLOCK:

This option will set the socket into a non blocking mode. The socket can be set for non-blocking only in conjunction with connect() system call. In

which case connect() call will not block until the connection is confirmed.

Using this option, a single process can place multiple calls without waiting

for connection establishment.

SIOC_WANPIPE_SOCK_STATE:

This commad will return 0 if socket is in CONNECTED state, return 1 if socket is in DISCONNECTED state, otherwise the return code will be 2. This command can be used after an OOB message to test whether the link is still up.

g. select() system call

int select(int sock_fh, fd_set *read, fd_set *write, fd_set *oob,

struct timeval *timeout)

Select() system call blocks and polls single or multiple sockets for receiving data, writing data or receiving OOB data. Select is the key factor in Sangoma secure socket architecture.

Select must be used to guarantee that no data will be lost.

The select method is tied to the socket flow control code, which will block if transmit buffers become full.  Please refer to the x25api example code.

h. send() system call

int send (int sockfh, void *buf, int len, unsigned int flags)

- Send() system call, can only be used when the connection is established, otherwise an error will be returned.

- If send is successful the return code will indicate the number of bytes transmitted.

- If send fails it means that the driver is busy: try again.   Note: it’s not an error condition.

- Every packet transmitted must contain a 16 byte Header (x25api_hdr_t, refer to the above data structure in section 5.f). The header data will not be passed out the port. It should be used to convey special information to the driver. For example, setting QDM bits.

i. recv() system call

int recv (int sockfh, void *buf, int len, unsigned int flags);

- Recv() system call, receives packets from the sock, if receive is successful the recv() will return number of bytes read. On error the return code is set to -1.

- Every receive packet comes with 16 byte (x25api_hdr_t refer to section 5.f), application should remove the header before using the data. Furthermore, the header will contain special bits which were transmitted by the remote switch, for  example QDM bits.

Details can be found in “X.25 SUPPORT SANGOMA CARDS”.

flags : set to zero, for regular data

set to MSG_OOB, for reading synchronous message

Note: The select() method will indicate that OOB message is waiting,

the recv() system call should be used with flags set to MSG_OOB.

Once the OOB message is received, use the packet type to

determine the asynchronous event.

## Sending and Receiving Data

- Once the connection is established, the socket is ready to receive and transmit data.

- Regardless of the application state, the driver will start sending data up the socket as soon as data arrives.

- It is up the application to wait for the data and receive packets from the socket.

- Once a packet is received by the user, the packet is flushed out of the socket; Thus, only one process should read a single socket a time.

- If the driver fills up the socket, receive interrupt will be turned off, which will block all the other channels from receiving data. If this occurs, after X number of seconds, the HDLC protocol will generate a protocol violation followed by the restart request, which will bring down all active channels. Therefore, make sure that the application is waiting to receive data all the time. Note, socket has a 64KB buffer and driver has 10 packet local buffer to prevent this from ever happening.

- In order to provide secure packet delivery to and from the application, the select() system call must be implemented. It will indicate when a data packet or OOB packet is pending, or when the socket is ready to transmit.

- The select must be setup to poll OOB events, otherwise the send() system call will fail when trying to send on a disconnected socket: in case of restart.

- Every transmitted packet must contain a 16 byte header which is to be used to convey important information to the driver : ex: QDM bits. Please refer to section 5.f, structure x25api_hdr_t.

- Every received packet has a 16 byte header which contains important information such as QDM bits. Please refer to section 5.f, structure x25api_hdr_t..

## X25API Programming Strategies

Most of the x25api application fall into ether server or client programming style.

- The server application usually waits for the incoming calls. On incoming call it ether spawns off children to handle the pending connection (server_v1.c), or handles the connection itself after which it returns to listen state (server_v2.c).  The later option is a more robust solution, especially in Linux multi-tasking environment. Under heavy-load conditions no incoming calls will

timeout, since most of the work is distributed to the children.

- On the other hand, client application usually requests the x25api connection. Upon successful connection, the client handles the call after which the session is terminated, or another connection is requested. (svc_clinet.c)

The two programming model implementations can be found in /usr/lib/wanpipe/api/x25 directory: server_v1.c and svc_client.c

We suggest that these programs be used as the starting point of the x25api application development. The user is free to modify change or use the code in any way which will provide the fastest time to market and ease of use.

The program xpipemon allows you to monitor the X.25 statistics, debug line problems and it will help you debug your programs. If you use the trace feature, you can see both your own data and the incoming data.

## APPENDIX

X25API TX/RX Header and IOCTL Command Data structure for the X25 API  system calls:  x25api_t:

typedef struct {

unsigned char qdm PACKED; /* Q/D/M bits */

unsigned char cause PACKED; /* cause field */

unsigned char diagn PACKED; /* diagnostics */

unsigned char pktType PACKED; /* packet type */

unsigned short length PACKED; /* data length */

unsigned char result PACKED; /* command result */

unsigned short lcn PACKED; /* bound lcn number */

char reserved[7] PACKED;

}x25api_hdr_t;

typedef struct {

x25api_hdr_t hdr PACKED;

char data[X25_MAX_DATA] PACKED;

}x25api_t;

i. QDM Byte

QDM Bits are bit mapped into one byte of data.

This filed is optional and should be set to zero in most cases.

Q: 	bit 2

Is a bit which marks a data packet as a special kind of packet.

Eg: Async PAD use the Q bit packets to negotiate PAD

parameters after call setup.

D: 	bit 1

Is used for confirmation of delivery of important packets.

M: 	bit 0

Used to fragment the messages which are larger than the maximum packet size.

### IMPORTANT

TX:  Use this byte to send fragmented data via Mbit option

RX:  Use this byte to determine if a fragment was received or a

whole packet.

IOCLT: When clearing call:  user must ensure that transmitted data was successfully received by the remote end.  By setting QDM = 0x80, the clear call will fail, if the x25 protocol has not transmitted all tx packets.  Thus, insuring that all packets were sent successfully.

ii. Cause and Diagnostic Fields

Cause and diagnostic fields are used to relay information to the remote user in regards to a asynchronous event. These fields are optional and should be set to zero in most cases.

Eg: When clear call is issued due to invalid call data, cause and

diagnostic fields should be set appropriately. Thus, when the

remote side receives an asynchronous message it will know the

reason for it.

iii. Packet Type Field

Packet type field should be used as read_only information. Upon receiving an OOB asynchronous message, the packet type field will indicate which asynchronous event occurred. Thus, based on the event, the state of the link can be established.

iv. Length Field

Length field indicates the size of the data buffer associated with the

command or the OOB event. Any time data buffer is used, the length field must be set the data buffer length.

v. Result Field

Result field should be used as read_only information. Result of every command executed will be stored in this field. This field should be used for statistic purposes only.

vi. LCN Field

This field should be used as read_only information. Upon successful link establishment the LCN field will indicate the link number currently used.