WANPIPETM

Multi-protocol WANPIPE Driver for Linux®

OPERATIONAL M A N U A L

Author: Nenad Corbic

Copyright (c) 1995-2002 Sangoma Technologies Inc.

# Introduction

At the heart of WANPIPE operation is the 'wanrouter' operation

script.  It is used to start/stop/restart and display

WANPIPE current status and environment.

# 'wanrouter' usage

/usr/sbin/wanrouter <cr>

WANPIPE: WAN Router startup script

Usage: wanrouter: {options} <wanpipe#> <interface>

wanrouter start : Starts all devices specified in

/etc/wanpipe/wanrouter.rc WAN_DEVICES

wanrouter stop  : Stops all devices specified in

/etc/wanpipe/wanrouter.rc WAN_DEVICES

wanrouter start wanpipe# : Start a single device

wanrouter stop  wanpipe# : Stops a single device

(# can range from 1 to 16)

wanrouter restart	 : Restart all devices specified in

/etc/wanpipe/wanrouter.rc WAN_DEVICES

wanrouter restart wanpipe# : Restart a single device

(# can range from 1 to 16)

wanrouter start wanpipe# if_name   : Start a single interface on device

wanrouter stop  wanpipe# if_name   : Stops a single interface on device

(# can range from 1 to 16)

wanrouter restart wanpipe# if_name : Restart a single interface on device

(# can range from 1 to 16)

wanrouter list		: List all active devices

wanrouter modules	: Show wanpipe kernel modules

wanrouter status	: Display status for all active devices

wanrouter summary	: Summary of config files in /etc/wanpipe

wanrouter hwprobe	: Display wanpipe hardware probe info.

wanrouter debug         : Check current wanpipe environment.

After a startup error run this command to

get a possible solution

i.e. wanrouter start; wanrouter debug;

wanrouter debug if_name : Display common debugging statistics

In case of line problems save to file,

wait 2-5mi and send to Sangoma Tech Support

i.e. wanrouter debug wp1fr16 > debug_file;

wanrouter messages      : Display wanpipe kernel event messages

i.e. /var/log/messages

wanrouter conflog       : Display wanpipe configuration parsing messages

i.e. /var/log/wanrouter

wanrouter version	: wanpipe version information.

# WANPIPE Environment Check

Automated environment check, can be performed using the wanrouter command.  Also after a startup failure wanrouter debug option will identify an error and offer possible solutions:

wanrouter debug

Make sure that the new WANPIPE kernel modules have been installed correctly and that they can be loaded into the kernel, and the hardware is detected.

/usr/sbin/wanrouter hwprobe

Check that a configuration file exists in /etc/wanpipe directory.

/usr/sbin/wanrouter summary

Make sure that no WANPIPE devices are currently running on the system

/usr/sbin/wanrouter list

# WANPIPE Startup/Shutdown

Please note that all kernel driver output messages are

located in /var/log/messages file.

Run 'tail -f /var/log/messages' in a separate console

window, to monitor WANPIPE start/stop and operation

messages.

To start/stop/restart WANPIPE devices listed in

/etc/wanpipe/wanrouter.rc file: (default is wanpipe1)

/usr/sbin/wanrouter start

/usr/sbin/wanrouter stop

/usr/sbin/wanrouter restart

To start/stop/restart single wanpipe device: (eg: wanpipe2)

This command is useful when running multiple devices

and would like to operate on a singe device.

/usr/sbin/wanrouter start wanpipe2

/usr/sbin/wanrouter stop wanpipe2

/usr/sbin/wanrouter restart wanpipe2

To start/stop/restart a single network interface from a

/etc/wanpipe/wanpipe1.conf configuration file:

This command is useful when adding an extra frame

relay DLCI. Also to reconfigure a single DLCI.

/usr/sbin/wanrouter start wanpipe2 wp2_fr18

/usr/sbin/wanrouter stop wanpipe2  wp2_fr18

/usr/sbin/wanrouter restart wanpipe2 wp2_fr18

To view current status and configuration of active

devices:

/usr/sbin/wanrouter status

# Error Debugging

## Startup/Shutdown Wanrouter Errors

After a startup failure run wanrouter debug

option for more information and possible solutions:

wanrouter debug

## Configuration Syntax Errors:

All wanpipe configuration file syntax errors are reported in /var/log/wanrouter

wanrouter conflogs

or

cat /var/log/wanrouter

## Kernel Driver Errors:

All device driver errors and events will be

displayed in /var/log/messages:

wanrouter messages

or

tail -f /var/log/messages

## Line Errors and Driver Statistics

After a successful WANPIPE startup any extra

debugging, such as line tracing or CSU/DSU

debugging will be done using:

/usr/sbin/wanpipemon –g

## Automated Line Debugging

For automated line debugging run the wanrouter debug if_name command.  It will display the most common statistics that deal with line problems.

wanrouter debug if_name  (where if_name is the name of wanpipe interface)

Detailed line debugging

Use the wanpipemon utility to view line and protocol ststistics and run line traces. For information on the commands, enter:

Wanpipemon

## Sangoma Tech Support

When reporting problems to Sangoma Tech Support please

dump the current system debug information into

a temporary file and send it via email.

1. wanrouter debug if_name > sangoma_debug_file.txt

(where if_name is wanpipe interface name: eg wp1fr16)

2. Wait for 2-5 minutes

3. Stop the debug by pressing ENTER

4. Email sangoma_debug_file.txt to Sangoma Tech Support

Please refer to README-4.debugging.