To upgrade SW on a DMG1000 or DMG2000 unit:

- If upgrading a DMG1000 unit to a Version 5.0 or greater software release, please verify that your DMG1000 hardware can support the upgrade. Your DMG1000 hardware can support the upgrade if it’s serial number is greater than or equal to the serial number listed in the Table below for the given DMG1000 model type:

| DMG1000 Model Name | Minimum Serial Number supporting V5.0 Software Upgrades |
| --- | --- |
| DMG1008DNI | IP006669 |
| DMG1008MTLDNI | IP006957 |
| DMG1008RLMDNI | IP006957 |
| DMG1008LS | IP006578 |

Note 1: All DMG2000 models support Version 5.0 or greater

Note 2: If your unit is currently loaded with a Version 4.0 based release (e.g. 4.0 Service Update 1 (SU1), 4.0 SU2, etc.), then you will need to upgrade to Version 5.0 SU1 before loading Version 5.1or greater. The 5.0 SU1 files and upgrade instructions can be found at http://www.dialogic.com/support/helpweb/mg/downloads.htm

- Copy the files to upgrade to a Windows workstation that can access the DMG1000/DMG2000 via the DMG1000/DMG2000 Web interface.

- Connect to the DMG1000/DMG2000 via the web interface and logon.

- Select the Upgrade web page from the Configure menu.

- Click the “Browse ...” button and select the appropriate *.fsh file for the model being upgraded (see Table below for list of files that need to be upgraded for each DMG1000/DMG2000 model).

- Click the Install button to upload the file. The DMG1000/DMG2000 will respond once the upgrade is complete. Do Not Restart or Power Cycle the unit.

- Select the Upgrade web page from the Configure menu.

- Click the “Browse ...” button to select the next file to upgrade (see Table below for list of files that need to be upgraded for each DMG1000/DMG2000 model)

- Click the Install button to upload the file. The DMG1000/DMG2000 will respond once the upgrade is complete.

  - Note: If you get an error during the upgrade, do not Restart or Power Cycle the unit. Simply retry the upgrade by starting over at step 8.

  - Repeat steps 7, 8, and 9 for each file you wish to upgrade. When all files have been upgraded, continue on with step 10.

- Select Restart from the Configure menu.

- Click the “Restart Now” button on the Restart page.

- The unit will restart using the new software.

| Model Name | Files | Comment |
| --- | --- | --- |
| DMG1008DNI | ami_6.0.SU8.0.003.app |  |
|  | ami_6.0.SU8.0.003.fsh |  |
|  | run_910FskEcho.dsp |  |
|  | iNim190.ilc |  |
|  | iNim653.iap |  |
|  | iNim60.ibt |  |
|  | ami_6.0.SU8.0.003.pkg |  |
|  | DNI_Cfg_Generic.ini | Download only if you want the default call progress tone |
|  | DNI_Cfg_Lucent.ini | Download only if connecting to an Avaya Definity |
|  | DNI_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 |
| DMG1004LS | ls_6.0.SU8.0.003.app |  |
|  | ls_6.0.SU8.0.003.fsh |  |
|  | run_910FskEcho.dsp |  |
|  | ls_6.0.SU8.0.003.pkg |  |
|  | LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E |
|  | LS_Cfg_Hicom300ECS.ini | Download only if connecting to an HiCom 300 E CS |
|  | LS_Cfg_LucentDefinity.ini | Download only if connecting to an Avaya Definity |
|  | LS_Cfg_Mitel.ini | Download only if connecting to a Mitel |
|  | LS_Cfg_Alcatel.ini | Download only if connecting to a Alcatel |
|  | LS_Cfg_ATTSys25.ini | Download only if connecting to an Avaya System 75 |
|  | LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX |
| DMG1008LS | ls_6.0.SU8.0.003.app |  |
|  | ls_6.0.SU8.0.003.fsh |  |
|  | run_910FskEcho.dsp |  |
|  | ls_6.0.SU8.0.003.pkg |  |
|  | LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E |
|  | LS_Cfg_Hicom300ECS.ini | Download only if connecting to an HiCom 300 E CS |
|  | LS_Cfg_LucentDefinity.ini | Download only if connecting to an Avaya Definity |
|  | LS_Cfg_Mitel.ini | Download only if connecting to a Mitel |
|  | LS_Cfg_Alcatel.ini | Download only if connecting to a Alcatel |
|  | LS_Cfg_ATTSys25.ini | Download only if connecting to an Avaya System 75 |
|  | LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX |
| DMG1008MTLDNI | mtl_6.0.SU8.0.003.app |  |
|  | mtl_6.0.SU8.0.003.fsh |  |
|  | run_910FskEcho.dsp |  |
|  | Mtl46.lca |  |
|  | mtl_6.0.SU8.0.003.pkg |  |
|  | MTLDNI_Cfg_Mitel.ini |  |
| DMG1008RLMDNI | mtl_6.0.SU8.0.003.app |  |
|  | mtl_6.0.SU8.0.003.fsh |  |
|  | run_910FskEcho.dsp |  |
|  | Mtl46.lca |  |
|  | mtl_6.0.SU8.0.003.pkg |  |
|  | RLMDNI_Cfg_ROLM.ini |  |
| DMG2030DTI | t1e1_6.0.SU8.0.003.app |  |
|  | t1e1_6.0.SU8.0.003.fsh |  |
|  | T1E1_MR17_7.msd |  |
|  | t1e1_6.0.SU8.0.003.pkg |  |
|  | T1_LS_Cfg_Avaya8500.ini | Download only if connecting to an Avaya 8500 using T1 CAS |
|  | T1_LS_Cfg_LucentG3.ini | Download only if connecting to an Lucent G3 using T1 CAS |
|  | T1_LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E using T1 CAS |
|  | T1_LS_Cfg_NEC2400IMX.ini | Download only if connecting to an NEC 2400 IMX using T1 CAS |
|  | T1_LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX using T1 CAS |
| DMG2060DTI | t1e1_6.0.SU8.0.003.app |  |
|  | t1e1_6.0.SU8.0.003.fsh |  |
|  | T1E1_ MR17_7.msd |  |
|  | t1e1_6.0.SU8.0.003.pkg |  |
|  | T1_LS_Cfg_Avaya8500.ini | Download only if connecting to an Avaya 8500 using T1 CAS |
|  | T1_LS_Cfg_LucentG3.ini | Download only if connecting to an Lucent G3 using T1 CAS |
|  | T1_LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E using T1 CAS |
|  | T1_LS_Cfg_NEC2400IMX.ini | Download only if connecting to an NEC 2400 IMX using T1 CAS |
|  | T1_LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX using T1 CAS |
| DMG2120DTI | t1e1_6.0.SU8.0.003.app |  |
|  | t1e1_6.0.SU8.0.003.fsh |  |
|  | T1E1_ MR17_7.msd |  |
|  | t1e1_6.0.SU8.0.003.pkg |  |
|  | T1_LS_Cfg_Avaya8500.ini | Download only if connecting to an Avaya 8500 using T1 CAS |
|  | T1_LS_Cfg_LucentG3.ini | Download only if connecting to an Lucent G3 using T1 CAS |
|  | T1_LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E using T1 CAS |
|  | T1_LS_Cfg_NEC2400IMX.ini | Download only if connecting to an NEC 2400 IMX using T1 CAS |
|  | T1_LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX using T1 CAS |
| DMG2060DTIS | t1e1_6.0.SU8.0.003.app |  |
|  | t1e1_6.0.SU8.0.003.fsh |  |
|  | T1E1_ MR17_7.msd |  |
|  | t1e1_6.0.SU8.0.003.pkg |  |
|  | T1_LS_Cfg_Avaya8500.ini | Download only if connecting to an Avaya 8500 using T1 CAS |
|  | T1_LS_Cfg_LucentG3.ini | Download only if connecting to an Lucent G3 using T1 CAS |
|  | T1_LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E using T1 CAS |
|  | T1_LS_Cfg_NEC2400IMX.ini | Download only if connecting to an NEC 2400 IMX using T1 CAS |
|  | T1_LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX using T1 CAS |
| DMG2120DTIS | t1e1_6.0.SU8.0.003.app |  |
|  | t1e1_6.0.SU8.0.003.fsh |  |
|  | T1E1_ MR17_7.msd |  |
|  | t1e1_6.0.SU8.0.003.pkg |  |
|  | T1_LS_Cfg_Avaya8500.ini | Download only if connecting to an Avaya 8500 using T1 CAS |
|  | T1_LS_Cfg_LucentG3.ini | Download only if connecting to an Lucent G3 using T1 CAS |
|  | T1_LS_Cfg_Hicom300E.ini | Download only if connecting to an HiCom 300 E using T1 CAS |
|  | T1_LS_Cfg_NEC2400IMX.ini | Download only if connecting to an NEC 2400 IMX using T1 CAS |
|  | T1_LS_Cfg_NEC2400IPX.ini | Download only if connecting to an NEC 2400 IPX using T1 CAS |

The *.pkg file is a package file that is a bundle of all of the runtime files associated with the new release for the specific model. This file allows the user to upgrade all of the runtime models (e.g. *.app, *,dsp, *.fsh, etc.) with the download of a single file. Note: This file can only be used when upgrading from a unit that is running Version 6.0 or greater.

Must load the ami_*.*, INim*.*, and run_910FskEcho.dsp files and restart prior to loading the DNI_Cfg_Lucent.ini file if upgrading from a version of DMG1000 Version 4 Service Update 3 or earlier.